# DATA SOURCES AND METHODOLOGY

## Forest Cover Change Detection - Nilgiris District

Project Team: Yarramsetti Jaswanth Venkat & Kanumuri V S L I Anooha
Date: October 2025
Study Period: 2000 - 2020

## 1. PRIMARY DATA SOURCES

### 1.1 Satellite Imagery

#### Landsat 5 Thematic Mapper (TM) - Year 2000

- Provider: United States Geological Survey (USGS)

- Access Platform: Google Earth Engine (GEE)

- Dataset ID: LANDSAT/LT05/C02/T1_L2

- Collection: Collection 2, Tier 1, Level-2 Surface Reflectance

- Temporal Coverage: January 1, 2000 to December 31, 2000

- Spatial Resolution: 30 meters

- Spectral Bands Used:

  - Band 3 (Red): 0.63-0.69 μm

  - Band 4 (Near-Infrared): 0.77-0.90 μm

- Scene Selection Criteria:

  - Cloud cover < 20%

  - Best available imagery for study area

  - Composite using median reducer

- Preprocessing: Atmospheric correction applied (Collection 2 processing)

#### Landsat 8 Operational Land Imager (OLI) - Year 2020

- Provider: United States Geological Survey (USGS)

- Access Platform: Google Earth Engine (GEE)

- Dataset ID: LANDSAT/LC08/C02/T1_L2

- Collection: Collection 2, Tier 1, Level-2 Surface Reflectance

- Temporal Coverage: January 1, 2020 to December 31, 2020

- Spatial Resolution: 30 meters

- Spectral Bands Used:

  - Band 4 (Red): 0.64-0.67 μm

  - Band 5 (Near-Infrared): 0.85-0.88 μm

- Scene Selection Criteria:

  - Cloud cover < 20%

  - Best available imagery for study area

  - Composite using median reducer

- Preprocessing: Atmospheric correction applied (Collection 2 processing)

Citation:

U.S. Geological Survey (2024). Landsat Collection 2 Level-2 Science Products. Earth Resources Observation and Science (EROS) Center. Available at: https://earthexplorer.usgs.gov/ Accessed via Google Earth Engine, October 2024.

### 1.2 Hydrological Data

#### JRC Global Surface Water Dataset

- Provider: Joint Research Centre, European Commission

- Access Platform: Google Earth Engine (GEE)

- Dataset ID: JRC/GSW1_3/GlobalSurfaceWater

- Temporal Coverage: 1984-2021

- Spatial Resolution: 30 meters

- Parameter Used: Water occurrence (percentage of time pixel is water)

- Threshold Applied: ≥ 50% occurrence = permanent water body

- Purpose: Identifying rivers, lakes, and water bodies for hydrology impact analysis

Citation:

Pekel, J.F., Cottam, A., Gorelick, N., Belward, A.S. (2016). High-resolution mapping of global surface water and its long-term changes. Nature, 540, 418-422. DOI: 10.1038/nature20584

### 1.3 Geographic Reference Data

#### Study Area Definition

- Region: Nilgiris District (Nīlakkal)

- States: Tamil Nadu and Karnataka, India

- Geographic Extent:

  - Longitude: 76.3°E to 76.9°E

  - Latitude: 11.15°N to 11.7°N

- Total Area: Approximately 3,500 square kilometers

- Coordinate System: WGS 84 (EPSG:4326)

- Elevation Range: ~300m to 2,600m above sea level

- Ecological Significance: UNESCO Biosphere Reserve, part of Western Ghats

#### Major Urban Centers (Coordinates)

- Ooty (Udhagamandalam): 76.695°E, 11.413°N - District Headquarters

- Gudalur: 76.497°E, 11.509°N - Major town, tea plantation region

- Coonoor: 76.796°E, 11.350°N - Hill station, tea production

- Kotagiri: 76.867°E, 11.421°N - Oldest hill station in Nilgiris

## 2. DERIVED DATA & CALCULATIONS

### 2.1 NDVI (Normalized Difference Vegetation Index)

Formula:

NDVI = (NIR - Red) / (NIR + Red)

Band Specifications:

- Landsat 5: NDVI = (SR_B4 - SR_B3) / (SR_B4 + SR_B3)

- Landsat 8: NDVI = (SR_B5 - SR_B4) / (SR_B5 + SR_B4)

Value Range: -1 to +1

- NDVI < 0: Water bodies, clouds

- 0 to 0.2: Barren land, urban areas

- 0.2 to 0.4: Sparse vegetation, grasslands

- 0.4 to 1.0: Dense vegetation, FOREST

Forest Classification Threshold:

- NDVI > 0.4 = Classified as Forest

- Rationale: Well-established threshold in literature for tropical/sub-tropical forests

References:

Tucker, C.J. (1979). Red and photographic infrared linear combinations for monitoring vegetation. Remote Sensing of Environment, 8(2), 127-150.

Rouse Jr, J.W., Haas, R.H., Schell, J.A., Deering, D.W. (1974). Monitoring vegetation systems in the Great Plains with ERTS. NASA Special Publication, 351(1974), 309.

### 2.2 Forest Change Detection Classes

Classification System:

| Code | Class Name | Definition | Color Code |
| --- | --- | --- | --- |
| 0 | No Forest | Non-forest in both 2000 and 2020 | White |
| 1 | Forest Loss | Forest in 2000, Non-forest in 2020 | Red |
| 2 | Forest Gain | Non-forest in 2000, Forest in 2020 | Light Green |
| 3 | Stable Forest | Forest in both 2000 and 2020 | Dark Green |

Change Detection Logic:

if (Forest_2000 == 1 AND Forest_2020 == 0) → Forest Loss

if (Forest_2000 == 0 AND Forest_2020 == 1) → Forest Gain

if (Forest_2000 == 1 AND Forest_2020 == 1) → Stable Forest

else → No Forest

### 2.3 Hydrological Impact Analysis

Buffer Analysis:

- Buffer Distance: 1,000 meters (1 km) around all water bodies

- Purpose: Identify deforestation in critical watershed zones

- Risk Assessment: Forest loss within buffer = HIGH RISK for:

  - Soil erosion into water bodies

  - Reduced water quality

  - Altered hydrology and runoff patterns

  - Sedimentation of rivers and reservoirs

Calculation:

- Total deforestation area (sq km)

- Deforestation within 1km of water (sq km)

- Percentage of high-risk deforestation = (Near-water deforestation / Total deforestation) × 100

## 3. PROCESSING PLATFORM & TOOLS

### 3.1 Google Earth Engine (GEE)

- Version: Code Editor (JavaScript API)

- Purpose: Primary analysis platform for satellite data processing

- Operations Performed:

  - Image collection filtering

  - Cloud masking

  - NDVI calculation

  - Change detection analysis

  - Statistical computations

  - Data export

### 3.2 QGIS (Quantum GIS)

- Version: 3.x (latest stable)

- Purpose: Cartographic visualization and map layout design

- Operations Performed:

  - Raster data visualization

  - Vector layer creation (boundaries, city points)

  - Map composition and layout

  - High-resolution map export

  - Professional cartographic output

### 3.3 Python (Optional)

- Version: 3.x

- Libraries: matplotlib, pandas, numpy

- Purpose: Statistical analysis and graph generation

## 4. DATA QUALITY & LIMITATIONS

### 4.1 Accuracy Considerations

- Spatial Resolution: 30m Landsat resolution may miss small-scale changes

- Temporal Resolution: Using annual composites (median) reduces seasonal variations

- Cloud Cover: Filtered to <20%, minimizes cloud contamination

- Classification Accuracy: NDVI threshold method ~85-90% accuracy for forest detection

### 4.2 Limitations

- Binary Classification: Only forest vs. non-forest (no forest type differentiation)

- Threshold Sensitivity: NDVI threshold of 0.4 may vary with local conditions

- Edge Effects: Mixed pixels at forest boundaries may cause classification errors

- Seasonal Variation: Deciduous forests may show seasonal NDVI fluctuation

### 4.3 Validation

- Visual comparison with high-resolution imagery (where available)

- Cross-reference with published forest cover statistics for Nilgiris

- Ground truth verification recommended for higher accuracy

## 5. ETHICAL & LEGAL CONSIDERATIONS

### 5.1 Data Usage Rights

- All datasets used are publicly available and free to access

- Google Earth Engine provides academic and research use access

- Proper citations provided for all data sources

- No proprietary or restricted data used

### 5.2 Open Science

- Methodology is reproducible using open-source tools

- Code can be shared for verification and replication

- Results contribute to public knowledge on forest conservation

## 6. REFERENCES

- U.S. Geological Survey (2024). Landsat Collection 2. Available: https://www.usgs.gov/landsat-missions/landsat-collection-2

- Pekel, J.F., et al. (2016). High-resolution mapping of global surface water. Nature, 540, 418-422.

- Gorelick, N., et al. (2017). Google Earth Engine: Planetary-scale geospatial analysis for everyone. Remote Sensing of Environment, 202, 18-27.

- Tucker, C.J. (1979). Red and photographic infrared linear combinations. Remote Sensing of Environment, 8(2), 127-150.

- Hansen, M.C., et al. (2013). High-resolution global maps of 21st-century forest cover change. Science, 342(6160), 850-853.

- UNESCO (2024). Nilgiri Biosphere Reserve. Available: https://en.unesco.org/biosphere/aspac/nilgiri

## 7. DATA AVAILABILITY STATEMENT

All data used in this study are publicly available through:

- Landsat Data: USGS Earth Explorer (https://earthexplorer.usgs.gov/)

- Water Data: Google Earth Engine Data Catalog

- Processing Platform: Google Earth Engine (https://earthengine.google.com/)

- GIS Software: QGIS (https://qgis.org/)

Contact for data queries:
Yarramsetti Jaswanth Venkat - RA2311003010656
Kanumuri V S L I Anooha - RA2311026010354

Document Version: 1.0
Last Updated: October 2025
Status: Final for Presentation